Preliminary draft of proposed

EU Ecolabel criteria for

Absorbent Hygiene Products

v1.4, 30 January 2013

Deadline for comments:

12 March 2013*

(*) Feedback received before the end of February will be possibly presented and discussed with Member States at the next EU Ecolabel Board meeting (Brussels, 6-8 March 2012).

Product scope:

1. The product group “adsorbent hygiene products” shall include products which:

- Are used for the physical and direct collection of human body waste streams and

- Are composed of a mix of natural fibres and polymers, with the fibre content lower than 90% by weight and

- Are disposable.

2. The product group shall comprise:

- all kinds of children’s diapers

- all kinds of sanitary pads/napkins and panty liners

- all kinds of tampons

- breast pads

3. The product group shall not comprise incontinence products and any other type of products falling under the scope of the Council Directive 93/42/EEC 14 June 1993 concerning medical devices.

Area of discussion 1 The Commission is collecting more detailed information about the classification and categorization used by industry for AHPs. A list of specific questions has been reported in the document "Summary of requirements and questions for stakeholders". Stakeholders are kindly invited to provide their written feedback to the Commission by 12 March 2013, at the latest. Feedback received before the end of February will be possibly presented and discussed with Member States at the next EUEB meeting (Brussels, 6-8 March 2012). These pieces of information would be important to: - Refine some of the environmental criteria presented in this document. - Understand the functional level at which the EU Ecolabel should be assigned (e.g. individual products with specific design and size characteristics; all AHP with same market name; all AHPs providing a specific function to the consumer) - Revise the scope of the product group (a) Which are the most relevant aspects considered for the classification of AHPs and for the identification of functionally equivalent types of products (e.g. size 3 taped diapers designed for night time use)? Parameter; Baby diapers; Panty liners; Feminine care pads; Tampons; Breast pads. Type and design (e.g. taped diapers designed for night time use). Size (e.g. size 3 diapers). Other aspects?. (b) Which is/are the most typical classification(s) used by industry to identify functionally equivalent types of AHPs on the market? (examples of some aspects of potential interest are preliminarily reported in the table below, please fill and modify the table if needed) Product; Classification and functional categorization used by industry; Information on weight (average weights and distribution). Baby diapers; Taped/pull on Time of use (Day/night time) Size (1, 2, 3, 4, 5, 6) …; …. Panty liners; Fluid handling (blood and urine, blood) …; …. Feminine care pads; Thickness (standard, ultra-thin, …) With/without wings …; …. Tampons; With/without applicators Applicator material (plastic, release paper, cardboard, …) Cotton/viscose tampons …; …. Breast pads; …; …. (c) In your opinion, what should be awarded the EU Ecolabel? Could you specify this for each product type, please? Options; Baby diapers; Panty liners; Feminine care pads; Tampons; Breast pads. A product with specific design and size produced by company X (e.g. a size 3 taped diaper designed for night time use). A specific type of AHPs produced by company Y (e.g. all the sizes of diapers with the same name and design). All the types of a specific AHP placed on the market by the company Z to fulfill a certain function (e.g. all the types of diapers used during the diapering period). Other options?.

Criteria Areas:

Table 1. Overview of criteria areas and individual criteria suggested for the EU Ecolabel of AHPs

| Criteria area | Proposed criteria |
| --- | --- |
| Materials and design | 1. Consumption of materials 2. Fluff pulp 3. Man-made fibres 4. Cotton 5. Polymers 6. Other materials |
| Chemicals | 7. Excluded or limited substances or mixtures |
| Manufacture | 8. Minimisation of the production waste |
| End-of-life | 9. Disposal of AHP |
| Fitness for Use | 10. Fitness for use and quality of the product |
| Other issues | 11. Information appearing on the EU Ecolabel 12. Social aspects |

Criterion 1: Consumption of materials

The weight and the composition thresholds reported in table Table 2 below shall be respected.

Table 3: Examples of potential weight and composition requirements for different AHPs

| Product |  | Weight threshold | Other requirements |
| --- | --- | --- | --- |
| Baby diapers |  | < 36 g (average value) | Cellulose content < 36.6% by weight (average value) |
| Panty liners |  | < 1.5 (average value) | - |
| Feminine care pads | Standard | < 10 (average value) | - |
|  | Ultra-thin | < 6 (average value) | - |
| Tampons |  | < 2.5 (average value) | - |
| Note: 1. Values reported are average figures and it would be appropriate to refine categorization and thresholds based on the feedback received by stakeholders. 2. Prescriptions could be appropriate for all the products within the scope or for only a portion of them. |  |  |  |

Assessment and verification: The applicant shall send a sample of the product including a declaration of compliance.

Area of discussion 2 Average data on product weights and compositions have been provided by EDANA (see Table 2). These could be used to set weight and composition thresholds for some of the AHPs being awarded with the EU Ecolabel. However, the question here is to understand: - For which categories of AHPs would such criteria make sense - Whether and how it would be appropriate to refer to average products or to differentiate per size, design, etc. - Whether alternative options can be followed A list of specific questions has been included in the document "Preliminary draft of proposed EU Ecolabel criteria for Absorbent Hygiene ". Stakeholders are kindly invited to provide their written feedback to the Commission by 12 March 2013, at the latest. Feedback received before the end of February will be possibly presented and discussed with Member States at the next EUEB meeting (Brussels, 6-8 March 2012). (a) Do you have any other data or any other source of data on the weight of different AHPs? Could this information be kindly shared with the Commission? Product; Data source; Weight distribution; Material composition range. Baby diapers. Panty liners. Feminine care pads. Tampons. Breast pads. (b) When setting weight thresholds, reference could be made to: Option; Answer. - Products with specific design and size (e.g. the average product within all the size 3 taped diapers designed for night time use)? If yes, how to better refine thresholds and criteria wording?. - Specific types of AHPs (e.g. the average product within all the diapers designed for night time use)? If yes, how to better refine thresholds and criteria wording?. - All the types of a specific AHPs placed on the market to fulfill a certain function (e.g. the average product within all the types of diapers used during the diapering period)? If yes, how to better refine thresholds and criteria wording?. - Other options?. (c) Which other possibilities exist in your opinion to design such criteria? Answer:.

Criterion 2: Fluff pulp

2.1) Sourcing

X% of pulp fibres (100?) shall be from pulp that has been grown according to the principles of Sustainable Forestry Management as defined by the UN FAO. The remaining 100-X% of pulp fibres shall be from pulp that is from legal forestry and plantations.

Assessment and verification: The applicant shall provide valid, independently certified chain of custody certificates demonstrating that pulp fibres have been grown according to Sustainable Forestry Management principles and/or are from legal sources. FSC and PEFC shall be accepted as independent certification schemes. Due diligence processes shall be followed as specified in Regulation (EC)19/2010 in order to minimise the risk that timber has been illegal harvested. Valid FLEGT or CITES licenses or third party certification shall be accepted as evidence of legal sourcing.

2.2) Bleaching:

(a) The pulp used in the product shall not be bleached with the use chlorine gas.

(b) The AOX emissions from the production of each kind of pulp shall not exceed 0.170 kg/ADT.

Assessment and verification:

(a) The supplier shall provide a declaration to the manufacturer that chlorine gas is not used.

(b) The applicant shall provide test reports using the test method AOX ISO 9562, accompanied by detailed calculations showing compliance with this criterion, together with related supporting documentation.

The supporting documentation shall include an indication of the measurement frequency.

AOX shall only be measured in processes where chlorine compounds are used for the bleaching of the pulp. AOX need not be measured in the effluent from non-integrated pulp production or in the effluents from pulp production without bleaching or where the bleaching is performed with chlorine-free substances.

Measurements shall be taken on unfiltered and unsettled samples either after treatment at the plant or after treatment by a public treatment plant.

The measurement period shall be 12 months of production. Measurements shall be effected on a weekly (or other?) basis.

In case of a new or a re-built production plant, measurements shall be effected on a weekly basis for a total of 8 consecutive weeks following steady running of the plant. The measurement shall be representative of the respective campaign.

2.3) Visual whitening and colouring agents

Visual whitening and colouring agents must not be intentionally added to the pulp.

Assessment and verification: The supplier shall provide a declaration to the manufacturer that the requirements have been fulfilled.

2.4) Emission of COD and phosphorous (P) to water and sulphur (S) compounds and NOx to air from production

The emissions to air and/or water from the pulp production shall be expressed in terms of points (PCOD, PS , PNOx , PP). Points are calculated by dividing actual emission by the reference values reported below.

| Pulp grade | Emissions (kg/ADT) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CODref | Sref | NOxrefe | Pref |
| Bleached chemical pulp (others than sulphite) | 18.0 | 0.6 | 1.6 | 0.045* |
| Bleached chemical pulp (sulphite) | 25.0 | 0.6 | 1.6 | 0.045 |
| CTMP | 15.0 | 0.2 | 0.3 | 0.005 |
| TMP/groundwood pulp | 3.0 | 0.2 | 0.3 | 0.01 |

* The emission value can be set up to 0.05 where it can be demonstrated that the higher level of P is due to P naturally occurring in the wood pulp

- None of the individual points PCOD, PS, PNOx, PP shall exceed 1.5.

- The total number of points (P total = PCOD + PS + PNOx + PP) shall not exceed 4.0.

Where different types of pulp are used, measured emissions and reference value shall be weighted according to the relative weight of each pulp type.

In case of a co-generation of heat and electricity at the same plant, the emissions of S and NOx resulting from electricity generation can be subtracted from the total amount.

The following equation can be used to calculate the proportion of the emissions resulting from electricity generation:

2 × (MWh(electricity)) / [2 × MWh(electricity) + MWh(heat)]

The electricity in this calculation is the electricity produced at the co-generation plant.

The heat in this calculation is the net heat value input produced at the co-generation plant and delivered to the pulp production process.

Assessment and verification: The applicant shall provide detailed calculations showing compliance with this criterion, together with related supporting documentation which shall include test reports using the following test methods: COD: ISO 6060; NOx: ISO 11564; S(oxid.): EPA no.8; S(red.): EPA no 16A; S content in oil: ISO 8754; S content in coal: ISO 351; P: EN ISO 6878, APAT IRSA CNR 4110 or Dr Lange LCK 349.

The supporting documentation shall include an indication of the measurement frequency and the calculation of the points for COD, S, NOx and P. It shall include all emissions of S and NOx which occur during the production of pulp, including steam generated outside the production site, except those emissions related to the production of electricity.

Measurements shall include recovery boilers, lime kilns, steam boilers and destructor furnaces for strong smelling gases. Diffuse emissions shall be taken into account.

Reported emission values for S to air shall include both oxidised and reduced S emissions (dimethyl sulphide, methyl mercaptan, hydrogen sulphide and the like). The S emissions related to the heat energy generation from oil, coal and other external fuels with known S content may be calculated instead of measured, and shall be taken into account.

Measurements of emissions to water shall be taken on unfiltered and unsettled samples either after treatment at the plant or after treatment by a public treatment plant.

The measurement period shall be 12 months of production. Measurements shall be effected on a weekly (or other?) basis.

In case of a new or a re-built production plant, measurements shall be effected on a weekly basis for a total of 8 consecutive weeks following steady running of the plant. The measurement shall be representative of the respective campaign.

2.5) Emissions of CO2 from production

CO2 emissions from non-renewable energy sources shall not exceed 1100 kg per tonne of pulp produced.

Reference values according to the following table shall be taken into account:

| Fuel | CO2 fossil emissions (g CO2fossil/MJ) |
| --- | --- |
| Coal | 95 |
| Crude oil | 73 |
| Fuel oil 1 | 74 |
| Fuel oil 2-5 | 77 |
| LPG | 69 |
| Natural Gas | 56 |
| Grid Electricity | 400 |

Assessment and verification: The applicant shall provide detailed calculations showing compliance with this criterion, together with related supporting documentation.

The applicant shall provide data on the air emissions of carbon dioxide. This shall include all sources of non-renewable fuels during the production of pulp, including the emissions from the production of electricity (whether on-site or off-site).

The measurement period shall be 12 months of production. Measurements shall be effected on a weekly (or other?) basis.

In case of a new or a re-built production plant, measurements shall be effected on a weekly basis for a total of 8 consecutive weeks following steady running of the plant. The measurement shall be representative of the respective campaign.

The amount of energy from renewable sources purchased and used for the production processes will not be considered in the calculation of the CO2 emissions: appropriate documentation that this kind of energy are actually used at the mill or are externally purchased shall be provided by the applicant.

2.6) Energy use during the production

Electricity

The electricity consumption related to the pulp production shall be expressed in terms of points (PE) as detailed below.

For each pulp i used, the related electricity consumption (Epulp,i  expressed in kWh/ADT) shall be calculated as follows:

Epulp,i = Purchased electricity (+ Internally produced electricity – sold electricity)

Points shall be calculated by dividing actual consumption figures by the reference values reported below.

Where different types of pulp are used, consumption figures and reference value shall be weighted according to the relative weight of each pulp type.

The number of points PE shall be less than or equal to 1.5.

Fuel (heat)

The fuel consumption related to the pulp production shall be expressed in terms of points (PF) as detailed below.

For each pulp i used, the related fuel consumption (Fpulp,i expressed in kWh/ADT) shall be calculated as follows

Fpulp,i = Purchased fuel (+ Internally produced fuel – sold fuel – 1.25 × internally produced electricity)

Fpulp,i (and its contribution to PF, pulp) does not need to be calculated for mechanical pulp unless it is marketed as air dried mechanical pulp containing at least 90% dry matter.

The amount of fuel used to produce the sold heat shall be added to the term ‘sold fuel’ in the equation above.

Points shall be calculated by dividing actual consumption figures by the reference values reported below.

Where different types of pulp are used, consumption figures and reference value shall be weighted according to the relative weight of each pulp type.

The number of points PF shall be less than or equal to 1.5.

Reference values according to the following table shall be taken into account.

| Pulp grade | Fuel (kWh/ADT) | Electricity (kWh/ADT) |
| --- | --- | --- |
| Chemical pulp | 4000* | 800 |
| Mechanical pulp | 900** | 1900 |
| CTMP | 1000 | 2000 |

*: For air dry market pulp (admp) containing at least 90% dry matter, this value may be upgraded by 25% for the drying energy

** This value is only applicable for admp

Assessment and verification: The applicant shall provide detailed calculations showing compliance with this criterion, together with all related supporting documentation. Reported details shall therefore include the total electricity and fuel consumption.

The applicant shall calculate all energy inputs, divided into heat/fuels and electricity used during the production of pulp and paper, including the energy used in the de-inking of waste papers for the production of recycled paper. Energy used in the transport of raw materials, as well as conversion and packaging, is not included in the energy consumption calculations.

Total heat energy includes all purchased fuels. It also includes heat energy recovered by incinerating liquors and wastes from on-site processes (e.g. wood waste, sawdust, liquors, waste paper, paper broke), as well as heat recovered from the internal generation of electricity — however, the applicant need only count 80% of the heat energy from such sources when calculating the total heat energy.

Electric energy means net imported electricity coming from the grid and internal generation of electricity measured as electric power. Electricity used for wastewater treatment need not be included.

Where steam is generated using electricity as the heat source, the heat value of the steam shall be calculated, then divided by 0.8 and added to the total fuel consumption.

In case of integrated mills, due to the difficulties in getting separate fuel (heat) figures for pulp and paper, if only a combined figure for pulp and paper production is available, the fuel (heat) values for pulp(s) shall be set to zero and the figure for the paper mill shall include both pulp and paper production.

2.7) Industrial best practices

The following measures shall be implemented in the pulp production plant order to limit emissions to water and air, consumption of resources and production of waste streams

| Environmental area | Measures |
| --- | --- |
| Water consumption and wastewater emissions | - Implementing water-saving solutions such as monitoring of water flow in a facility and water recirculation in closed systems, including the stripping of contaminated steam condensates and the reuse of condensates and white water in the process - Separating water loops for each process units (e.g. pulping, bleaching) to prevent the carry-over of pollutants to subsequent process steps and to restrict the organic load of process water - Preferring catalytical disinfection with hydrogen peroxide to the use of biocides for the control of the growth of microorganisms - Implementing multi-step waste water treatment plants for decreasing the emissions of AOX and unchlorinated toxic organic compounds - Using low chlorine dioxide charge bleaching sequences with partial recycling of process water, where elemental chlorine-free (ECF) bleaching is in place. |
| Waste management | - Implementing an integrated waste management plan to optimize prevention, reuse, recycling, recovery, and final disposal of waste according to waste hierarchy. - Separating different waste fractions to allow reuse or recirculation of the single fractions. - Recycling fibres, wherever possible |
| Air emissions | - Treating flue gases with electrostatic precipitators (ESP) and fabric filters (FF) to prevent excessive dust emissions. - Implementing a system to optimize and to monitor combustion conditions of all combustion steps (e.g. recovery boilers, soda boilers or lime kilns) in order to reduce air emissions. - Washing and filtrating lime mud in the kraft pulping process in order to reduce the hydrogen sulfide emissions in the re-burning process |
| Energy management | - Implementing measures to optimize energy efficiency (e.g. via segregation of hot and cold waste water streams prior to heat recovery and recovery of heat from the hot stream) and to reduce the consumption of fossil fuels. - Implementing on-site generation of electricity and heat in combined heat and power plants (CHP), which can save up to 30% of energy when compared to conventional technologies. - Implementing measures to optimize energy efficiency (e.g. via segregation of hot and cold waste water streams prior to heat recovery and recovery of heat from the hot stream) and to reduce the consumption of fossil fuels - Implementing on-site generation of electricity and heat in combined heat and power plants (CHP), which can save up to 30% of energy when compared to conventional technologies. |
| Note: This criterion is still in a preliminary form. Stakeholders are invited to indicate which measures would lead to the most significant environmental benefits, would be technically feasible and, thus, would be worthy of consideration for the final criteria proposal. |  |

Assessment and verification: The supplier shall provide a declaration to the manufacturer that the requirements have been fulfilled.

Criterion 3: Man-made cellulose fibres (including viscose, modal, lyocell, cupro, triacetate)

3.1) Sourcing

(a) 25% of pulp fibres shall be from pulp that has been grown according to the principles of Sustainable Forestry Management as defined by the UN FAO. The remaining 75% of pulp fibres shall be from pulp that is from legal forestry and plantations.

(b) Dissolving pulp produced from cotton linters shall meet with the requirements 4.1 and 4.2 for cotton (sourcing and traceability).

Assessment and verification:

(a) The applicant shall provide valid, independently certified chain of custody certificates demonstrating that pulp fibres have been grown according to Sustainable Forestry Management principles and/or are from legal sources. FSC and PEFC shall be accepted as independent certification schemes. Due diligence processes shall be followed as specified in Regulation (EC)19/2010 in order to minimise the risk that timber has been illegal harvested. Valid FLEGT or CITES licenses or third party certification shall be accepted as evidence of legal sourcing.

(b) The application shall provide evidence of compliance according to criteria 4.1 and 4.2 for cotton.

3.2) Bleaching

(a) The pulp used to manufacture fibres shall not be bleached with the use of chlorine gas.

(b) The resulting level of halogenated compounds (OX) in the fibres shall not exceed 0.150 kg/ADT

Assessment and verification:

(a) The supplier shall provide a declaration to the manufacturer that chlorine gas is not used.

(b) The applicant shall provide a test report, using the following test method: ISO 11480.1997 (controlled combustion and microcoulometry).

3.3) Visual whitening and colouring agents

Visual whitening and colouring agents must not be intentionally added to the pulp.

Assessment and verification: The supplier shall provide a declaration to the manufacturer that the requirements have been fulfilled.

3.4) Production of fibres

(a) The following limits shall be respected in the viscose and in the modal fibres production process:

| Fibre type | Sulphur emissions to air Limit value (g/kg) | Zinc emissions to water Limit value (g/kg) |
| --- | --- | --- |
| Staple fibre | 30 | 0.30 |
| Filament fibre - Batch washing - Integrated washing | 60 170 | 0.16 0.16 |
| Note: Limit values expressed as annual average |  |  |

(b) For cupro fibres, the copper content of the effluent water leaving the site, expressed as an annual average, shall not exceed 0.10 ppm.

(c) More than 50% of pulp used to manufacture fibres shall be obtained from dissolving pulp mills that recover value from their spent process liquor either by 1) generating on-site electricity and steam and/or 2) by manufacturing chemical co-products.

Assessment and verification:

(a), (b) The applicant shall provide detailed documentation and/or test reports showing compliance with this criterion, together with a declaration of compliance.

(c) The applicant shall provide a list of pulp suppliers used to make the fibres and the proportion they supply. Supporting documentation and evidence shall be provided that the required proportion of suppliers have energy generating equipment and/or co-product recovery and manufacturing systems installed at production sites.

3.5) Industrial best practices

The following measures shall be implemented in the pulp production plant order to limit emissions to water and air, consumption of resources and production of waste streams

| Environmental area | Measures |
| --- | --- |
| Water consumption and wastewater emissions | - Removal of Na2SO4 from wastewater (spinning baths, in which the viscose solution is pressed through spinnerets) for coagulation of the fibres - Reduction of Zinc from wastewaters by alkaline precipitation followed by sulphide precipitation. - Use of anaerobic sulphate reduction techniques for sensitive waterbodies. If further desulphurization is necessary, anaerobic reduction to H2S must be carried out. - Use of separate effluent collection systems for - Contaminated process effluent water - Potentially contaminated water from leaks and other sources, including cooling water and surface runoff from process plant areas, etc. - Uncontaminated water |
| Waste management | - Use of fluidized bed incinerators to burn non-hazardous wastes with subsequent heat and energy recovery - Recycling of fibres, wherever possible |
| Air emissions | - Condensation of exhaust air from spinning streets to recover CS2 and backcycling into the process. (different technologies available). - Operation of spinning frames in houses in order to minimise CS2 emissions, (spinning frames are the sources of CS2 emissions). Housings have to be equipped with leak-proof sliding windows and have suction systems inside where excess CS2 is purged to a recovery facility. - Application of exhaust air desulphurization processes based on catalytic oxidation with H2SO4 production. |
| Energy management | - Implementing measures to optimize energy efficiency (e.g. via segregation of hot and cold waste water streams prior to heat recovery and recovery of heat from the hot stream) and reduce the consumption of fossil fuels. - Applying on-site generation of electricity and heat in combined heat and power plants (CHP), which can save up to 30% of energy when compared to conventional technologies |
| Note: This criterion is still in a preliminary form. Stakeholders are invited to indicate which measures would lead to the most significant environmental benefits, would be technically feasible and, thus, would be worthy of consideration for the final criteria proposal. |  |

Assessment and verification: The supplier has to provide a declaration to the manufacturer that the requirements have been fulfilled.

Criterion 4: Cotton and other natural cellulosic seed fibres

4.1) Sourcing

Cotton and other natural cellulosic seed fibres (hereinafter referred to as cotton) shall be grown according to one of the following two production standards and must meet the common content claim requirements.

Option 1: IPM

All cotton used shall be grown according to Integrated Pest Management (IPM) principles as defined by the UN FAO’s IPM programme and shall not be grown using any of the following substances:

Alachlor, aldicarb, aldrin, campheclor (toxaphene), captafol, chlordane, 2,4,5-T, chlordimeform, chlorobenzilate, cypermethrin, DDT, dieldrin, dinoseb and its salts, endosulfan, endrin, glyphosulfate, heptachlor, hexachlorobenzene, hexachlorocyclohexane (total isomers), methamidophos, methyl-o-dematon, methylparathion, monocrotophos, parathion, phosphamidon, pentachlorophenol, thiofanex, triafanex, triazophos

Cotton shall not contain more than 0.5 ppm in total (sensitivity of the test method permitting) of the substances listed above. Cotton is not required to be tested where it has been certified by a suitable IPM scheme that prohibits the use of the listed substances.

Option 2: Organic

A minimum of 50% of cotton shall be grown according to the requirements laid down in Regulation (EC) No 834/2007 or the US National Organic Programme (NOP).  The cotton content may include organically grown cotton and transitional organic cotton.

Assessment and verification:

Option 1: The applicant shall provide evidence that the cotton is grown by farmers that participate either in Government IPM programmes or third party certified IPM schemes.  Government programmes include the UN FAO IPM programme, the USDA IPM programme and other programmes to be specified. Certification to the following IPM schemes will be accepted – the Better Cotton Initiative (BCI), Cotton Made in Africa and the Australian Better Management Programme (BMP) – together with IPM schemes with equivalent restrictions on pesticide use.

A test report should be provided demonstrating that the listed substances have not been used.  The following test methods shall be used, as appropriate:

- US EPA 8081 A (organo-chlorine pesticides, with ultrasonic or Soxhlet extraction and apolar solvents (iso-octane or hexane)),

- 8151 A (chlorinated herbicides, using methanol),

- 8141 A (organophosphorus compounds),

- 8270 C (semi-volatile organic compounds).

Tests should be made on raw cotton, before it comes through any wet treatment, for each lot of cotton or two randomly selected samples a year if more than two lots of cotton per year are received.

Declarations of non-use compiled from farmer producer groups will be accepted where they are verified by annual site visits.

Option 2: Organic content should be certified by an independent organisation to have been produced in conformity with the production and inspection requirements laid down in Regulation 834/2007/EC or the US National Organic Programme (NOP).  Verification either on an annual basis for a proportion of the cotton purchased or of the blending of cotton at the spinning stage shall be accepted.

4.2) Traceability

It shall be possible to trace the cotton used to manufacture the product from farmers to fabric production. This shall be ensured for all cotton purchased. Documentary evidence shall be provided that assures the integrity of the cotton content claim.

Assessment and verification: Transaction records and/or invoices shall be provided that document the quantity of cotton purchased on an annual basis from farms up until fabric production before dyeing, printing and finishing.  Documentary evidence shall reference the Control Body or certifier of the cotton.  Cotton certified to the GOTS, Fair Trade, OE Blended and OE 100 standards, as well as any other equivalent content claim standards shall be accepted as complying with these requirements.

4.3) Bleaching

Cotton shall not be bleached with the use of chlorine gas.

Assessment and verification: The supplier shall provide a declaration to the manufacturer that chlorine gas is not used.

4.4) Visual whitening and colouring agents

Visual whitening and colouring agents must not be intentionally added to cotton materials

Assessment and verification: The supplier shall provide a declaration to the manufacturer that the requirements have been fulfilled.

4.5). Industrial best practices

The following measures shall be implemented in the pulp production plant order to limit emissions to water and air, consumption of resources and production of waste streams

| Environmental area | Measures |
| --- | --- |
| Water consumption and wastewater emissions | - Implementing water-saving solutions such as monitoring of water flow in a facility, adjustment of processes in pretreatment to quality requirements in downstream processes and re-use of water. - Implementing a monitoring plan in order to avoid/ minimize any kind of surplus of applied chemicals and auxiliaries (e.g. by automated dosing and dispensing of chemicals) and to minimize consumption of complexing agents in hydrogen peroxide bleaching. - Implementing multi-step waste water treatment plants to decrease the emission of AOX. |
| Waste management | - Implementing an integrated waste management plan to optimize prevention, reuse, recycling, recovery, and final disposal of waste according to waste hierarchy. - Separating different waste fractions to allow reuse or recirculation of the single fractions. |
| Air emissions | - Proving that in the selection of auxiliaries and chemicals within the facility higher preferences are given to products with a low volatility and low smell intensity. |
| Energy management | - Implementing measure to optimize energy efficiency (e.g. via segregation of hot and cold waste water streams prior to heat recovery and recovery of heat from the hot stream) and to reduce the consumption of fossil fuels. - Applying on-site generation of electricity and heat in combined heat and power plants (CHP), which can save up to 30% of energy when compared to conventional technologies. |
| Note: This criterion is still in a preliminary form. Stakeholders are invited to indicate which measures would lead to the most significant environmental benefits, would be technically feasible and, thus, would be worthy of consideration for the final criteria proposal. |  |

Assessment and verification: The supplier has to provide a declaration to the manufacturer that the requirements have been fulfilled.

Criterion 5: Polymers

5.1) Sourcing*

An X% by weight of the polymers shall come from renewable feedstock*.

Assessment and verification: See note*

* Note: This criterion will be proposed only if practical assessment and verification schemes and/or procedures are found which can be used to provide evidence that specific renewable-based polymers are functionally equivalent to petroleum-based materials and do not lead to worse environmental performance or critical trade-offs.

5.2) Heavy metals / organostannic compounds

Contents of lead, cadmium, mercury, hexavalent chrome and attendant impurities as well as organostannic compounds must be lower than 0.1% of the mass of the respective material (e.g. plastic) in the product.

Assessment and verification: The supplier has to provide a declaration to the manufacturer that the requirements have been fulfilled.

5.3) SAP

(a) Super Absorbent Polymers may contain a maximum of 400 ppm residual monomers (total of unreacted acrylic acid and cross linkers).

(b) SAP may furthermore as a maximum contain 5% (weight/weight) of water-soluble extracts (i.e. monomers and oligomers of acrylic acid with lower molecular weight than SAP and salts)

Assessment and verification:

(a) The manufacturer must document the composition of the superabsorbent by means of a product safety data sheet which specifies the full name and CAS number and the residual monomers contained in the product classified in accordance with the above requirements and the quantities thereof. The methods used for the analyses must be described and the names of the laboratories used for analysis must be stated. The recommended test methods are WSP 210.2 (05), ERT 410.2 (02)/IST 210.2(02), ISO 17190 – 2:2001

(b) The manufacturer must specify the quantity of water-soluble extracts in the super-absorbents. The methods of analysed used must be described and the analysis laboratories must be stated. The recommended test method is WSP 270.2 (05), ERT 470.2 (02)/IST 270.2(02), ISO 17190 – 10:2001Error: Reference source not found.

5.4) Industrial best practices

The following measures shall be implemented in the pulp production plant order to limit emissions to water and air, consumption of resources and production of waste streams

| Environmental area | Measures |
| --- | --- |
| Water consumption and wastewater emissions | - Implementing water-saving solutions such as monitoring of water flow in a facility and circulating the water in closed systems. |
| Waste management | - Implementing an integrated waste management plan to optimize prevention, reuse, recycling, recovery, and final disposal of waste according to waste hierarchy. - Separating different waste fractions to allow reuse or recirculation of the single fractions. |
| Air emissions | No measure identified |
| Energy management | - Implementing measures to optimize energy efficiency. - Reusing the steam generated during the manufacture of SAPs (e.g. at Verbund sites) |
| Note: This criterion is still in a preliminary form. Stakeholders are invited to indicate which measures would lead to the most significant environmental benefits, would be technically feasible and, thus, would be worthy of consideration for the final criteria proposal. |  |

Assessment and verification: The supplier has to provide a declaration to the manufacturer that the requirements have been fulfilled.

Criterion 6: Other materials

6.1) Adhesive materials

Adhesives must not contain:

  - Colophony resins,

  - Diisobutyl phthalate (DIBP, CAS 84-69-5) or

  - Formaldehyde (50-00-0).

The requirement does not apply if these substances

- are not intentionally added to the material or to the final product, and

- are present in the final product in concentrations below 100 ppm (0.01% by weight) in the final product.

For formaldehyde, the maximum limit for the content of formaldehyde generated during adhesive production is 250 ppm, measured in newly produced polymer dispersion. Content of free formaldehyde in hardened adhesive (glue) must not exceed 10 ppm. Hotmelt adhesives are exempted from this requirement.

Assessment and verification: The supplier shall provide a declaration to the manufacturer that the requirements have been fulfilled. Test results for formaldehyde shall be provided.

6.2) Inks and dyes

(a) The product and any homogeneous part of it must not be dyed. This prescription does not apply to tampon strings, packaging materials, tape, titanium dioxide in polymers and viscose is exempted from this requirement.

Materials that are not directly in contact with the skin may, however, be dyed if the dye has a special function (e.g. dying of nursing pads to reduce visibility of the product through white or light coloured clothing).

(b) Inks and dyes must comply with criterion 9 on the implementation of Article 6(6) and Article 6(7) of the Regulation (EC) No 66/2010.

Assessment and verification:

(a) The supplier has to provide a declaration to the manufacturer that the requirements have been fulfilled. In case dyes are used, their presence will be justified by indicating the special function provided. Competent bodies will decide if the presence of the dye is necessary.

(b) The supplier has to provide a declaration to the manufacturer that the requirements have been fulfilled.

6.3) Lotions and fragrances

(a) Products intended for infants, babies and children under the age of three year shall be fragrance-free. Infant, baby and/or children products refers to products that are marketed as designed and intended for infants, babies and/or children or have any of these words on the label/packaging.

(b) Any ingoing substance added to the product as a fragrance shall be manufactured and handled following the code of practice of the International Fragrance Association (IFRA). The code can be found on IFRA website: http://www.ifraorg.org. The recommendations of the IFRA Standards concerning prohibition, restricted use and specified purity criteria for materials shall be followed by the manufacturer. In addition, the following fragrances shall not be used in AHPs

| Common name | CAS number |
| --- | --- |
| Cinnamal | 104-55-2 |
| Cinnamyl Alcohol* | 104-54-1 |
| Citral | 5392-40-5 |
| Coumarin | 91-64-5 |
| Eugenol* | 97-53-0 |
| Farnesol* | 4602-84-0 |
| Geraniol* | 106-24-1 |
| Hydroxycitronellal | 107-75-5 |
| Hydroxyisohexyl 3-cyclohexene carboxaldehyde (HICC) | 31906-04-4 |
| Isoeugenol* | 97-54-1 |
| Limonene (oxidised) | 5989-27-5 |
| Linalool* (oxidised) | 78-70-6 |
| Oak moss | 90028-68-5 |
| Tree moss | 90028-67-4 |
| CANANGA ODORATA and Ylang-ylang oil | 83863-30-3; 8006-81-3 |
| EUGENIA CARYOPHYLLUS LEAF / FLOWER OIL | 8000-34-8 |
| JASMINUM GRANDIFLORUM / OFFICINALE | 84776-64-7; 90045-94-6; 8022-96-6 |
| MYROXYLON PEREIRAE (Balsam of Peru) | 8007-00-9 |
| SANTALUM ALBUM (Sandelholz) | 84787-70-2; 8006-87-9 |
| TURPENTINE (oil) | 8006-64-2; 9005-90-7; 8052-14-0 |
| * including their respective esters |  |

(c) In case a product contains lotions or fragrances, the manufacturer must declare its presence on the packaging.

Assessment and verification: (a), (b), (c) The applicant shall provide a declaration of compliance.

6.4) Silicone

a) Where components of the product are treated with silicone, the manufacturer must ensure that employees are protected from the solvents.

b) Neither octamethyl cyclotetrasiloxane D4 (CAS 556-67-2) nor decamethyl cyclopentasiloxane D5 (CAS 541-02-6) may be present in chemical products used in the silicone treatment of components. The requirement does not apply if D4 and D5:

- are not intentionally added to the material or to the final product, and

- are present in the final product in concentrations below 100 ppm (0.01% by weight)

Assessment and verification:

a) The supplier shall provide information on the method used for the treatment of silicone and documentation attesting that employees are protected.

b) The supplier shall provide a declaration to the manufacturer that the requirement has been fulfilled.

Area of discussion 3 Stakeholders are invited to analyse the criteria presented for each materials and to answer to the specific questions reported in the document "Proposed EU Ecolabel criteria for Absorbent Hygiene – Preliminary Draft". Stakeholders are kindly invited to provide their written feedback to the Commission by 12 March 2013, at the latest. Feedback received before the end of February will be possibly presented and discussed with Member States at the next EUEB meeting (Brussels, 6-8 March 2012). Feedback will be used as material to update and complete the presented set of criteria. (a) Which requirements on materials are considered more significant and meaningful for the development of EU Ecolabel criteria? Is there any modification or additional requirement you would like to propose? Answer:. (b) Would it make sense from an environmental point of view to promote the use of polymers produced from renewable resources? If yes, what should be the minimum percentage of polymers produced from renewable resources? In which practical ways could the sustainability of bio-plastic be verified and/or certified? Answer:. (c) Would you have any additional recommendations? Answer:.

Criterion 7: Excluded or limited substances or mixtures

a) Substances and mixtures of relevance for Article 6(6) of the EU Ecolabel Regulation

According to the Article 6(6) of Regulation (EC) No 66/2010 on the EU Ecolabel, the product or any homogeneous part of the product shall not contain substances meeting criteria for classification with the hazard statements or risk phrases specified in the table below in accordance with Regulation (EC) No 1272/2008 or Directive 67/548/EC nor shall it contain substances referred to in Article 57 of Regulation (EC) No 1907/2006.

Table 3. List of hazard statements and risk phrases

| Hazard Statement | Risk Phrase |
| --- | --- |
| H300 Fatal if swallowed | R28 |
| H301 Toxic if swallowed | R25 |
| H304 May be fatal if swallowed and enters airways | R65 |
| H310 Fatal in contact with skin | R27 |
| H311 Toxic in contact with skin | R24 |
| H330 Fatal if inhaled | R26 |
| H331 Toxic if inhaled | R23 |
| H340 May cause genetic defects | R46 |
| H341 Suspected of causing genetic defects | R68 |
| H350 May cause cancer | R45 |
| H350i May cause cancer by inhalation | R49 |
| H351 Suspected of causing cancer | R40 |
| H360F May damage fertility | R60 |
| H360D May damage the unborn child | R61 |
| H360FD May damage fertility. May damage the unborn child | R60; R61; R60-61 |
| H360Fd May damage fertility. Suspected of damaging the unborn child | R60-R63 |
| H360Df May damage the unborn child. Suspected of damaging fertility | R61-R62 |
| H361f Suspected of damaging fertility | R62 |
| H361d Suspected of damaging the unborn child | R63 |
| H361fd Suspected of damaging fertility. Suspected of damaging the unborn child | R62-63 |
| H362 May cause harm to breast fed children | R64 |
| H370 Causes damage to organs | R39/23; R39/24; R39/25; R39/26; R39/27; R39/28 |
| H371 May cause damage to organs | R68/20; R68/21; R68/22 |
| H372 Causes damage to organs through prolonged or repeated exposure | R48/25; R48/24; R48/23 |
| H373 May cause damage to organs through prolonged or repeated exposure | R48/20; R48/21; R48/22 |
| H400 Very toxic to aquatic life | R50 |
| H410 Very toxic to aquatic life with long-lasting effects | R50-53 |
| H411 Toxic to aquatic life with long-lasting effects | R51-53 |
| H412 Harmful to aquatic life with long-lasting effects | R52-53 |
| H413 May cause long-lasting harmful effects to aquatic life | R53 |
| EUH059 Hazardous to the ozone layer | R59 |
| EUH029 Contact with water liberates toxic gas | R29 |
| EUH031 Contact with acids liberates toxic gas | R31 |
| EUH032 Contact with acids liberates very toxic gas | R32 |
| EUH070 Toxic by eye contact | R39-41 |
| H334: May cause allergy or asthma symptoms or breathing difficulties if inhaled | R42 |
| H317: May cause allergic skin reaction | R43 |

Substances or mixtures which change their properties through processing (e.g., become no longer bioavailable, or undergo chemical modification in a way that removes the previously identified hazard) are exempted from the above requirement.

Concentration limits for substances and mixtures which may be, or have been, assigned the hazard statements or risk phrase listed above or which meet the criteria for classification in the hazard classes or categories, and concentration limits for substances meeting the criteria of Article 57 (a), (b) or (c) of Regulation (EC) No 1907/2006, shall not exceed the generic or specific concentration limits determined in accordance with the Article 10 of Regulation (EC) No 1272/2008. Where specific concentration limits are determined they shall prevail over the generic ones.

Concentration limits for substances meeting criteria set out in Article 57(d), (e) or (f) of Regulation (EC) No 1907/2006 shall not exceed 0.1% weight by weight.

Concentrations are referred to the product and to homogeneous parts of the product in which substances can be found.

(b) Substances listed in accordance with Article 59(1) of Regulation (EC) No 1907/2006

No derogation from the prohibition set out in point (a) Article 6(6) of Regulation (EC) No 66/2010 shall be granted concerning substances identified as substances of very high concern and included in the list provided for in Article 59 of Regulation (EC) No 1907/2006, present in the product in concentrations higher than 0.1%. Specific concentration limits determined in accordance with Article 10 of Regulation (EC) No 1272/2008 shall apply where the concentration is lower than 0.1%.

Assessment and verification:

(a) Applicants must provide evidence that no substances classified according to art. 6(6) and 6(7) of the EU Ecolabel regulation is contained in their products.

The applicant shall provide the exact composition of the product and of each homogeneous part of it. Compliance with this criterion shall be demonstrated by providing a declaration on the non-classification of each substance into any of the hazard classes associated to the hazard statements referred to in the above list in accordance with Regulation (EC) 1272/2008, as far as this can be determined, as a minimum, from the information meeting the requirements listed in Annex VII of Regulation (EC) 1907/2006. This declaration shall be supported by summarized information on the relevant characteristics associated to the hazard statements referred to in the above list, to the level of detail specified in section 10, 11 and 12 of Annex II of Regulation (EC) 1907/2006 (Requirements for the Compilation of Safety Data Sheets).

Information on intrinsic properties of substances may be generated by means other than tests, for instance through the use of alternative methods such as in vitro methods, by quantitative structure activity models, or by the use of grouping or read-across in accordance with Annex XI of Regulation (EC) 1907/2006. The sharing of relevant data is strongly encouraged.

The information provided shall relate to the forms or physical states of the substance or mixtures as used in the final product.

For substances listed in Annexes IV and V of REACH, exempted from registration obligations under Article 2(7)(a) and (b) of Regulation 1907/2006 REACH, a declaration to this effect will suffice to comply with the requirements set out above.

(b) The list of substances identified as substances of very high concern and included in the candidate list in accordance with Article 59 of Regulation (EC) No 1907/2006 can be found at:

http://echa.europa.eu/chem_data/authorisation_process/candidate_list_table_en.asp

Reference to the list shall be made on the date of application. The applicant shall provide a declaration of compliance with this criterion, together with related documentation, such as declarations of compliance signed by the material suppliers and copies of relevant Safety Data Sheets for substances or mixtures.

Area of discussion 4 Stakeholders are invited to analyse the criterion presented and to answer to the specific questions reported in the document "Proposed EU Ecolabel criteria for Absorbent Hygiene – Preliminary Draft". Stakeholders are kindly invited to provide their written feedback to the Commission by 12 March 2013, at the latest. Feedback received before the end of February will be possibly presented and discussed with Member States at the next EUEB meeting (Brussels, 6-8 March 2012). Feedback will be used as material to update and complete the presented set of criteria. (a) Could there be a variation of composition associated with different sizes of the same product? How could this affect the criteria development process, in particular with reference to content of any substances in the product? Product; Answer. Baby diapers. Panty liners. Feminine care pads. Tampons. Breast pads. (b) Which are the main SAPs used, their market share and their main sources of concern (if any)? Answer:. (c) Is sodium polyacrylates classified as H412? Is the presence of this material essential in AHP? Which are the alternatives? Answer:.

Criterion 8: Minimisation of Production waste

The amount of production waste that is not reused within the AHP manufacturing process or not converted to useful materials and energy shall not exceed 0.5% by weight of the end product.

Assessment and verification: The manufacturer shall provide evidence of the amount of waste that cannot be reused within the AHP manufacturing process or that is not converted to materials and energy.

### Criterion 9: Guidance on the product disposal

The producers shall write on the packaging that the product must be disposed in waste bins and not flushed into the toiled. This requirement shall apply only to feminine pads and tampons.

Assessment and verification: The manufacturer shall provide visual evidence of the packaging energy.

Criterion 10: Fitness for use and quality of the product

The efficiency/quality of the product must be satisfactory and must at the least match that of equivalent products on the market.

Fitness-for-use has to be tested with respect to the characteristics and parameters reported below. Performance thresholds must be matched, where these have been identified.

Table 4. Fitness-for-use characteristics and test methods

| Characteristic and parameter |  | Scope | Tests | Performance threshold |
| --- | --- | --- | --- | --- |
| User tests | Overall performance | All AHP | - User trial | 90% of the consumers testing the product shall rate themselves as "satisfied" (rating 4) or "very satisfied" (rating 5) in a rating scale from 1 to 5. |
|  | Leakage protection | All AHP | - User trial | Leakage results in less than 10% of all diaper changes. |
|  | Skin dryness and compatibility | All AHP | - User trial | 90% of the consumers testing the product shall rate themselves as "satisfied" (rating 4) or "very satisfied" (rating 5) in a rating scale from 1 to 5. |
|  | Fit and comfort | All AHP | - User trial | 90% of the consumers testing the product shall rate themselves as "satisfied" (rating 4) or "very satisfied" (rating 5) in a rating scale from 1 to 5. |
| Safety tests | Product safety | All AHP | - Product safety testing process | not available |
|  | Chemical safety | All AHP | - Tests in accordance with the Oeko-Tex Standard 100 | not available |
|  | Microbiological safety | All AHP | - Tests in accordance with the European Pharmocopoeia | not available |
| Technical tests | Absorption | Diapers | - Absorption rate - Absorption before leakage | not available |
|  | Skin dryness and compatibility | Diapers | - Skin wetting (rewet) | not available |

Assessment and verification:

A test report must be provided including a description of test methods, test results and data used.

The test methods used must be based as much as possible on product-relevant, reproducible and rigorous methods. Tests must be carried out by laboratories and institutes qualified in the considered field of activity and their objectivity must be recognised. There must be a financial independence between the testing organisations and the manufacturers.

Sampling, test design, panel recruitment and the analysis of test results must comply with ASTM E1958-07e1. Tests should be conducted on the main product designs and/or the most common size. Special care must be taken regarding sampling, transport and storage of the products to guarantee comparable results. It is recommended not to blind products or repack them in neutral packaging due to the risk of altering the performance of products and/or packaging.

Information on the test methods used must be made available to all relevant stakeholders, for instance on the company website. The results must be presented in language, units and symbols that are understandable to the consumers. The presentation of the test results must be clearly explained. It must include the criteria used to select the products tested, the representativeness and the sampling of the products, the characteristics selected and if applicable, the reasons why some were not included, the test methods used and their limitations if any. External factors such as branding, market shares and advertising that may have an impact on the perceived performance of the products should be communicated. Clear guidelines on the use of test results must be provided (for example, it should be required to indicate the date and source of the test result).

Additional requirements for user tests:

- Consumer surveys must be conducted and analysed according to standard statistical practices, i.e. ASTM E1958-07e1

- The recommended number of required answers in a user panel is at least 30

- The results are to be statistically evaluated after the user trial has been completed

- Each product should be assessed on the basis of a questionnaire compiled by the test institute. The test is to last at least 72 hours per test, a full week when possible

- The ratio of male to female individuals should be 1:1 (not applicable to products designed specifically for one gender)

- All participants should be current users of the specific type/size of diaper being tested

- A mixture of participants representing proportionally different groups of consumers available on the market should take part in the study

- The product should be used under direct supervision of the respondents, in the same way and conditions as the product they normally use.

- If the test is conducted in a different country than the target market, the name of the country should be clearly stated

- Sick individuals those with a chronic skin condition should not participate in the test. In cases where individuals become ill during the course of the user trial, this is to be indicated on the questionnaire and the results are not to be taken into consideration for the assessment.

Additional requirements for safety tests:

- Evidence that a product safety testing process is in place shall be provided.

- Chemical tests shall be carried out in accordance with the Oeko-Tex Standard 100

- The determination of the microbiological quality shall be carried out on the original product in accordance with the European Pharmacopoeia. As applicable, other guidelines, recommendations, relevant legal decisions, scientific publications and other regulations and standards shall also be taken into consideration.

Additional requirements for technical tests:

- Tests can be conducted with saline solution (0.9% NaCl analytical grade in de-ionized water)

- A minimum of 5 samples should be tested, and results should be reported with the average and standard deviation from those 5 samples.

- A description of the construction of the diaper should be recorded, together with the weight and dimensions of the diaper.

Area of discussion 5 Additional information need to be collected in order to revise this criterion on "Fitness for use and quality of the product". In particular, it would be important to understand: - which performance characteristics are of importance for which product (and worthy of inclusion in the criterion) - which assessment and verification procedure to follow - for which characteristics it is possible to set performance thresholds (a) Do you think that the approach outlined above could work (fusion of Nordic Swan requirements, EDANA guidelines, stakeholers consultation)? If not, how the criterion could be improved? Answer:. (b) For each of the AHPs within the scope of the EU Ecolabel, which are the most important characteristics and parameters in terms of quality and performance and the related testing methods? Product; Characteristic and parameter of relevance; Testing methods; Performance threshold; Guidelines and cost estimate. Baby diapers. Panty liners. Feminine care pads. Tampons. Breast pads. (c) Can performance thresholds be revised/defined for some of the characteristics and parameters reported above? Can guidelines and cost estimates on test methods be provided? Characteristic and parameter; Testing methods; Performance threshold; Guidelines and cost estimate. User tests; Overall performance. Leakage protection. Skin dryness and compatibility. Fit and comfort. Safety tests; Product safety. Chemical safety. Microbiological safety. Technical tests; Absorption. Skin dryness and compatibility.

Criterion 11: Information appearing on the EU Ecolabel

The use of the EU Ecolabel logo is protected in primary EU law. The logo should be visible and legible. The EU Ecolabel registration/license number must appear on the product, it must be legible and clearly visible.

The optional label with text box shall contain the following text:

- The product satisfies the most relevant performance and quality tests;

- The use of substances of concern for human health and environment is restricted;

- The product is designed in order to reduce the impact from the consumption of resources

The guidelines for the use of the optional label with text box can be found in the "Guidelines for use of the Ecolabel logo" on the website: http://ec.europa.eu/environment/ecolabel/documents/logo_guidelines.pdf

The following text should moreover appear on the packaging:

"For more information on why this product has been awarded the EU Ecolabel, please visit http://ec.europa.eu/environment/ecolabel/".

Assessment and verification: The applicant shall provide a sample of the product label, together with a declaration of compliance with this criterion

Criterion 12: Consideration of social aspects

Applicants shall ensure that the fundamental principles and rights at work as specified in the - International Labour Organisation’s Core Labour Standards shall be observed by all production sites used to manufacture EU Ecolabelled products.  The ILO Core Standards are:

029 Forced Labour

087 Freedom of Association and Protection of the Right to Organise

098 Right to Organise and Collective Bargaining

100 Equal remuneration

105 Abolition of Forced Labour

111 Discrimination (Employment and Occupation)

138 Minimum Age Convention

182 Elimination of the Worst Forms of Child Labour

Assessment and verification: The applicant shall obtain reports on compliance from their production sites and from the productions sites of their suppliers. These should be compiled and provided to Competent Bodies. Third party certification will be accepted as evidence of compliance. A license may be suspended or revoked if substantive evidence is received that ILO Core Labour Standards have been breached.

Area of discussion 6 (a) Would you have any additional recommendations for the criteria development process? Answer:.

As provided for in Regulation (EC) No 1272/2008 of the European Parliament and of the Council

As provided for in Council Directive 67/548/EEC